Product List

- Teensy 4.0 Teensy 4.0 | Buy in Australia | DEV-15583 | Core Electronics

- Adafruit VL53L4CX Time of Flight Distance Sensor Adafruit VL53L4CX Time of Flight Distance Sensor - ~1 to 6000mm [STEMMA QT / Qwiic] : Adafruit Industries, Unique & fun DIY electronics and kits

- Arduino Compatible 9G Micro Servo Motor SG90 Arduino Compatible 9G Micro Servo Motor | Jaycar Australia

- Adafruit 9-DOF Orientation IMU Fusion Breakout - BNO055 Adafruit 9-DOF Absolute Orientation IMU Fusion Breakout - BNO055 | Buy in Australia | ADA2472 | Core Electronics

- Adafruit APDS9960 Proximity, Light, RGB, and Gesture Sensor Adafruit APDS9960 Proximity, Light, RGB, and Gesture Sensor | Little Bird Electronics

- MG996R Servo 360° Rotation (Continuous) MG996R Servo 360° Rotation (Continuous) - Maker Store PTY LTD

|  | Name | Pros | Cons | Input Voltage (V) | Accuracy |
| --- | --- | --- | --- | --- | --- |
| Product 1 | 6 DOF Sensor – (MPU6050) | Small size and low power | Has as much DOF as spike brick and has drift error | 3V – 5V | Pitch: ±1.0° Roll: ±1.0° Yaw: No absolute accuracy limit. |
| Product 2 | 9 DOF Sensor – fusion breakout – (BNO085) | Better navigation and saves CPU power | Yaw drift and needs complex math | 3V – 5V | Yaw: ±3.0° Roll: ±1.0° Pitch: ±1.0° |
| Product 3 | 10 DOF Mems IMU Sensor | Full orientation and height data | Magnetic interference and uses more battery | 3V – 8V | Pitch: ±2.0° to ±3.0° Roll: ±2.0° to ±3.0° Yaw: ±3.0° to ±5.0° |
| Product 4 | 9 DOF BNO055 | On chip fusion and absolute orientation. | Heat drift and high-power use. | 3.3V – 5V | Pitch: ±1.0° to ±2.0° Roll: ±1.0° to ±2.0° Yaw: ±3.5° |

IMU

|  | Name | Pros | Cons | Min Range (mm) | Max Range (mm) | Input Voltage (V) | Accuracy | Speed (Hz) | Measurement Angle (deg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product 1 | Pololu (VL53L0X ) Carrier | Small breadboard footprint and Wide 2.6V to 5.5V tolerance | Older 1st-generation tech and 30 mm close-range blind spot | 30mm - 40mm | 2000mm | 3.3V | ±3% (at 1200mm) | 50Hz per reading | 25 |
| Product 2 | Adafruit 5396 (VL53L4CD) | Reads right down to 1 mm and fast 100 hz rate | Higher cost per mm of range and shortest max range (1.3m) | 1mm | 1200mm | 3.3V | ±3% (at >200mm) | 100Hz per reading | 22 |
| Product 3 | PiicoDev (VL53L1X) | Easy daisy-chain system and great micro python support | 30 mm close-range blind spot and strictly 3.3V (No 5V tolerance) | 40mm | 4000mm | 3.3V – 5V | ~±1% | 50Hz per reading | 27 |
| Product 4 | Adafruit 5425 (VL53L4CX) | Tracks multi-objects at once and reads right down to 1 mm | Massive driver code size and crashes standard Arduino Uno | 0 – 10mm | 6000mm | 2.6V – 3.5V | ~±5% | 30Hz per reading | 18 |

Distance

Motors

|  | Name | Pros | Cons | Input Voltage (V) | Stall Torque (Nm) | No Load Speed (rpm) |
| --- | --- | --- | --- | --- | --- | --- |
| Product 1 | Maker Store (MG996R) 360° Continuous | Durable internal metal gears and uses standard, simple PWM code | Centre-Point Drift and no angle control | 4.8V – 7.2V | 1.8Nm at 6V and 0.92Nm at 4.8V | 58.8rpm |
| Product 2 | Tribotix Dynamixel (XC430-W150-T) | True 360° absolute position control and live feedback (temp, torque load, speed) | Complex Infrastructure and Requires a specialized serial interface board | 6.5V – 14.8V (12 recommended) | 1.6Nm at 12V | 106rpm |

Colour

|  | Name | Pros | Cons | Input Voltage (V) | Detection Distance (mm) |
| --- | --- | --- | --- | --- | --- |
| Product 1 | Adafruit 1334 (TCS34725) | Integrated IR-blocking filter and Massive 3,800,000:1 dynamic range | Discontinued by Adafruit and Legacy design lacks solderless STEMMA QT quick connectors | 3.3V – 5V | 3mm – 10mm |
| Product 2 | Fermion (TCS3200) | High-resolution light-to-frequency square wave output and Programmable colour filters and full-scale output frequency | No onboard IR filter and requires heavy manual software calibration to interpret raw pulses | 2.7V – 5.5V | 10mm |
| Product 3 | Adafruit (APDS-9960) | 4-in-1 multi-sensor (RGB, Ambient Light, Proximity, and Gestures) and Built-in UV/IR blocking filters for cleaner environmental data | Fixed I2C address (0x39) makes it hard to run multiples together and RGB colour sensor accuracy is lower than dedicated colour ICs | 2.4V – 3.6V | Not specified |

Servo

|  | Name | Pros | Cons | Input Voltage (V) | Stall Torque |
| --- | --- | --- | --- | --- | --- |
| Product 1 | Feetech (FT90R) | Continuous 360 rotation and Digital driver handles quick responses | No position control (cannot hold an angle) and requires deadband calibration to stop | 4.8V – 6V | 1.3 kg·cm at 4.8V 1.5 kg·cm at 6V |
| Product 2 | Tower Pro (SG90) | Precise position control up to 160° and connects directly to basic boards | Limited physical movement span and Plastic gears wear down under stress | 3.5V – 6V | 1.6 kg·cm at 4.8V 1.9 kg·cm at 6V |

Microcontroller

|  | Name | Pros | Cons | Processing Speed | Memory |
| --- | --- | --- | --- | --- | --- |
| Product 1 | Teensy 4.0 | Fast processing speed and small size | Fails instantly with 5V and no built in SD slot | 600 MHz | 1024 KB RAM 2 MB Flash |
| Product 2 | Teensy 4.1 | Built in micro-SD and ethernet and massive I/O capability. | Fails instantly with 5V, high power consumption and is bigger than 4.0 | 600 MHz | 1024 KB RAM 8 MB Flash |
| Product 3 | Arduino Mega 2560 | Native 5V logic compatibility and is very plug and play | Very slow computing, bulky, heavy and not much memory space | 16 MHz | 8 KB RAM 256 KB Flash |